# Beyond Backscatter: InSAR coherence from detected SAR images

The paper is “Beyond Backscatter: InSAR coherence from detected SAR images” by Francescopaolo Sica, Andrea Pulella, and Michael Schmitt. Its central idea is quite important: use deep learning to predict interferometric coherence from two detected SAR intensity/backscatter images, without doing the usual sub-pixel SLC coregistration and interferometric processing.

1. What is the paper trying to solve?

Normally, if we want InSAR coherence, we need two SLC images, accurately coregister them, and then calculate coherence from their complex-valued signals.

That is computationally expensive, particularly for large-scale processing.

The authors ask:

Can we estimate coherence directly from the backscatter/intensity information in two SAR images?

Their answer is yes, to a surprisingly good degree, using a deep-learning regression model.

The important conceptual point is that they do not claim to reconstruct the missing interferometric phase. Instead, the neural network learns a statistical relationship:

paired SAR backscatter → expected interferometric coherence

2. The key idea

The pipeline is essentially:

SAR image 1 + SAR image 2
↓
radiometric calibration / preprocessing
↓
Residual U-Net
↓
predicted coherence map

The ground-truth coherence is generated separately from accurately coregistered SLC data.

So during training:

SLC pair → conventional coherence → training target

while the neural network only sees:

backscatter image 1 + backscatter image 2 → predicted coherence

This is a very useful supervised-learning setup because they can obtain large amounts of training labels automatically from conventional InSAR processing.

3. Why can backscatter contain information about coherence?

This is probably the most important scientific insight to take from the paper.

The authors explicitly acknowledge that there is no direct analytical relationship between detected backscatter intensity and interferometric coherence. Coherence depends on several factors:

- SNR

- temporal decorrelation

- geometric decorrelation

- volume scattering

- azimuth/range spectral differences

- scattering mechanisms

- acquisition geometry

- scene stability

So a simple formula like

backscatter intensity → coherence

is not sufficient.

Instead, the neural network learns more complicated cues from the relationship between the two acquisitions.

The paper identifies several useful cues:

- temporal consistency of speckle statistics

- local structural similarity

- texture stability

- radiometric differences between acquisitions

- scattering behavior

That is a major takeaway for our project.

4. What model did they use?

They use a Residual U-Net.

The architecture has:

- encoder

- decoder

- residual blocks

- skip connections

- max pooling in the encoder

- upsampling in the decoder

- final 1×1 convolution for coherence prediction

The U-Net aspect is important because coherence is a spatially detailed pixel-level quantity.

The network needs both:

- increasingly abstract/contextual features, and

- preservation of fine spatial structures.

The skip connections help retain those spatial details.

Training configuration

The paper reports:

| Parameter | Value |
| --- | --- |
| Patch size | 128 × 128 |
| Batch size | 64 |
| Epochs | 20 |
| Optimizer | Adam |
| Initial LR | 0.0001 |
| Loss | MSE |
| Training/validation | 90% / 10% |

5. The most important experiment: two images vs one image

This is one of the strongest parts of the paper.

They train the network in two configurations:

Model A

Image 1 + Image 2 → coherence

Model B

Image 1 → coherence

The single-image model performs substantially worse.

For the San Francisco Bay experiment:

- baseline estimator: RMSE = 0.275

- proposed dual-input model: RMSE = 0.108

- single-input model: RMSE = 0.261

This is important because it demonstrates that the network isn't simply learning:

"urban = high coherence, vegetation = low coherence."

Instead, it is actually learning information from the relationship between the two SAR acquisitions.

This is something I would definitely carry into our project.

If our project involves predicting some temporal/interferometric property, do not design the model around a single SAR image unless there is a strong reason to do so.

The temporal pair itself contains valuable information.

6. Generalization across temporal baselines

This is another particularly interesting result.

The model was trained only using 12-day Sentinel-1 pairs.

But they tested it at:

- 0 days

- 6 days

- 12 days

- 36 days

And it still worked.

Their RMSE for the overall region was:

| Temporal baseline | RMSE |
| --- | --- |
| 0 day | 0.068 |
| 6 day | 0.106 |
| 12 day | 0.108 |
| 36 day | 0.116 |

The authors interpret this as evidence that the model is learning general relationships associated with temporal decorrelation, rather than simply memorizing "12 days."

This has a major implication.

For our project, we should explicitly test temporal generalization rather than only random train/test splits.

A random split could put nearly identical scenes/acquisitions into both training and test sets and give us an overly optimistic result.

A much stronger experiment is:

train on certain temporal separations → test on unseen temporal separations.

7. Geographic generalization

The model was trained primarily using Sentinel-1 data over Italy.

They then tested it in very different environments:

- San Francisco Bay

- Atacama Desert

- Brazilian rainforest

- Swiss Alps

The model retained useful performance across these environments.

The reported RMSEs for three geographic scenarios were:

- Atacama: 0.055

- Rondonia: 0.105

- Schwyz Alps: 0.139

The degradation in the Alps is particularly informative because the authors associate it with strong geometric/topographic effects.

Lesson for our project

Geographic generalization needs to be a first-class experiment.

Don't just randomly divide pixels from the same geographic region into training and testing.

Instead:

Train region A → test region B

That tells us whether the model has learned the phenomenon or simply learned the visual characteristics of the training region.

8. Cross-polarization and cross-sensor experiments

The authors go further.

The model was trained primarily using Sentinel-1 VV.

They tested it on VH data.

It still worked, although performance degraded somewhat.

They also tested on ALOS L-band HH data.

Again, the model retained meaningful predictive capability, but performance dropped:

RMSE = 0.215

This is interesting because it suggests that the learned representation isn't completely sensor-specific.

But it also gives an important warning:

Cross-sensor generalization is possible, but domain adaptation/fine-tuning will probably be necessary for high performance.

The authors themselves suggest sensor/frequency-specific fine-tuning.

9. One of the biggest practical contributions: GRD / Google Earth Engine

This may be the most practically useful aspect of the paper.

Normally, InSAR processing requires SLC data.

But the authors demonstrate transfer to geocoded Sentinel-1 GRD data from Google Earth Engine.

GRD data has undergone:

- multilooking

- terrain correction

- interpolation

- reprojection

- other processing

So its statistical properties are different from SLC-derived data.

They therefore perform domain adaptation/fine-tuning.

Their adaptation strategy is relatively simple:

- spatial multilooking

- approximate GRD spatial resolution/statistics

- radiometric normalization

- fine-tune the model

The resulting model produces useful coherence predictions on the geocoded GRD data.

10. What I think is the biggest contribution of the paper

The contribution isn't simply:

"We used a U-Net to predict coherence."

That would undersell the paper.

The more important contribution is:

The statistical information necessary to estimate coherence is partially encoded in paired SAR intensity observations, even after the complex phase information has been discarded.

The network can recover useful information about:

temporal stability + scattering behavior + texture consistency + speckle evolution

from the two detected images.

That is the conceptual insight I would carry forward.

11. Important limitations

The paper is quite clear about its limitations.

1. It doesn't replace conventional InSAR

The network isn't recovering the actual interferometric phase.

It's predicting coherence statistically.

So it cannot replace a conventional interferometric pipeline when accurate physical interferometric measurements are required.

2. Some decorrelation mechanisms aren't observable from intensity

This is perhaps the most important limitation.

If a decorrelation mechanism doesn't leave a sufficiently strong signature in the detected backscatter, the network can't magically recover it.

This becomes particularly problematic under extreme geometric conditions.

3. No uncertainty estimation

The output is deterministic:

predicted coherence = one number per pixel.

There is no confidence interval or uncertainty map.

4. Ground truth itself comes from conventional coherence estimation

Therefore, the model can inherit biases/artifacts from the reference coherence estimator used to generate its labels.

This is a very important issue if we're building our own ML system.

12. How relevant is this to our project?

Assuming our project is the SAR/InSAR deep-learning work we've been discussing, I would put this paper in the highly relevant / foundational-methodology category.

Not because we should simply reproduce it, but because it gives us a useful framework for several design decisions.

Relevance breakdown

| Component | Relevance |
| --- | --- |
| SAR preprocessing | Very high |
| Paired temporal SAR inputs | Very high |
| Deep-learning architecture | High |
| Coherence prediction | Very high |
| Temporal generalization | Very high |
| Geographic generalization | Very high |
| Domain adaptation | Very high |
| GRD/GEE processing | High |
| Cross-sensor transfer | High |
| Uncertainty estimation | Opportunity for us |
| Physics-informed modeling | Opportunity for us |

13. What we should pick up for our project

I would extract seven concrete ideas from this paper.

A. Use paired SAR observations

This is probably the single most important takeaway.

Instead of:

SAR image → target

consider:

SAR image at t₁ + SAR image at t₂ → target

because the difference/relationship between acquisitions contains temporal information that a single image cannot provide.

The paper's ablation experiment strongly supports this.

B. Use a U-Net-type architecture as a strong baseline

For a pixel-wise SAR prediction task, their Residual U-Net is a very reasonable baseline.

I would actually structure our experiments as:

Baseline 1: conventional CNN
Baseline 2: U-Net
Baseline 3: Residual U-Net
Baseline 4: our proposed architecture

That gives us a defensible experimental progression.

C. Make temporal generalization part of the evaluation

Don't only report:

training accuracy / random test accuracy.

Instead test:

train: 12-day
test: 6-day / 12-day / 24-day / 36-day

or whatever temporal intervals are appropriate for our dataset.

This paper gives us a strong precedent for doing this.

D. Make geographic holdout testing

We should ideally have:

Training regions ≠ testing region

rather than randomly selecting pixels from the same geographic scene.

For example:

Train: Region A + B + C
Validation: Region D
Test: completely unseen Region E

This will make our result much more convincing.

E. Treat domain shift explicitly

This paper demonstrates that SLC and GRD distributions are different.

That's a general ML lesson for SAR:

Changing processing level can be a domain shift.

Likewise:

- VV → VH

- C-band → L-band

- SLC → GRD

- slant range → geocoded

- one geographic region → another

should be considered potential domain shifts.

We should measure these rather than assume the model will transfer automatically.

F. Preserve the spatial resolution

The paper repeatedly emphasizes that traditional intensity-based coherence estimators require spatial averaging, which reduces resolution.

The neural network can produce high-resolution predictions while retaining spatial structures.

For our project, that supports using an architecture capable of maintaining fine spatial information, rather than aggressive downsampling.

G. Go beyond what this paper does: uncertainty + physics

This is where I think we can potentially make our project more interesting.

The paper itself says future work should investigate:

- uncertainty-aware formulations

- cross-sensor adaptation

- physics-informed learning

Those are essentially open opportunities.

For example, rather than predicting only:

ρ

we could potentially predict:

ρ, σρ

where the second output represents model/measurement uncertainty.

Or incorporate physically meaningful information such as:

- temporal baseline

- perpendicular baseline

- incidence angle

- polarization

- wavelength

- SNR

- land-cover/scattering information

alongside the SAR images.

That would make the model more physically informed than the paper's purely image-based formulation.

14. A possible architecture for our project

Based on this paper, I would consider something like:

SAR(t1) ───────┐

│

├──> Siamese / dual encoder

SAR(t2) ───────┘

│

↓

Feature fusion

│

+ baseline / metadata

│

↓

Residual U-Net

│

↓

┌──────────┴──────────┐

↓                     ↓

Target map           Uncertainty map

This would retain the strongest idea from the paper—the paired observations—while extending it with physically meaningful metadata and uncertainty.

15. One thing I would NOT blindly copy

I wouldn't simply take their:

Sentinel-1 VV → Residual U-Net → MSE

and call that our methodology.

That's a good baseline, but the paper itself shows where the weaknesses are:

- extreme geometric conditions

- cross-sensor domain shift

- uncertainty

- dependence on reference coherence

- limited physical interpretability

Those weaknesses give us opportunities to differentiate our project.

16. The paper in one paragraph

In one sentence:
The paper demonstrates that interferometric coherence can be predicted at high spatial resolution from paired detected SAR backscatter images using a Residual U-Net, without requiring precise SLC coregistration.

The model is trained using conventional SLC-derived coherence as supervision, but only receives backscatter images. It substantially outperforms a conventional intensity-based coherence estimator, benefits strongly from having both temporal acquisitions as input, and generalizes across temporal baselines, geographic regions, polarization, acquisition geometry, and even frequency to some extent. The approach can also be adapted to globally available Sentinel-1 GRD data in Google Earth Engine. However, it remains a statistical approximation of coherence, cannot recover information that is absent from detected intensity, does not provide uncertainty estimates, and can degrade under strong domain shifts or extreme geometric conditions.

My main takeaway for our project

I would extract this research strategy from the paper:

Use paired SAR observations to learn temporal/scattering relationships, use conventional InSAR products as supervision where available, deliberately test generalization across time and geography, and then improve on the paper by incorporating physical acquisition parameters, uncertainty, and/or domain adaptation.

That gives us a very solid starting point without simply duplicating the paper.

Most importantly, I think the paper can serve as one of our key baseline/reference papers: we can explicitly position our work as “building on Sica et al.'s detected-SAR-to-coherence regression framework, but addressing X”, where X is the particular problem our project is targeting.